Exporting Data from Landmark and Importing into Geolog (Time Depth)

In Landmark - > Well Data Management, make sure that you have time-depth data to export for a particular well.

Data -> Export -> Well Data Export

Export Name -> change to /mking/d1/Projects/AUGER/Data/Time_Depth directory and name this file something like jv_426_1_td.asc

Export Type = Ascii

Format/Mapping File: /opt/ow2003/data/wlx/Auger_TimeDepth.wlx, you may have to edit this format file to export all the necessary information. For help with this, see Heather, once you have tried it on your own.

Carto System: UTM_15_Auger (make sure this matches your project)

Measurement System: US Oil Field

Depth Mode will be greyed to TVD

Well Lists: All Wells

Well Names: Select ONE well name that you want to start with

Well catagories: Select Well Header

Curve Catagories: Select Time Depth Table

At the top menu -> Options -> Time Depth, make sure all your choices are selected, there may be more than one.

Controls -> Export

Open a text editor and view the new file (jv_426_1_td.asc). Notice that if the time-depth data has more than one curve, they are listed in this one file, one right after the other (scroll down to view)! Geolog will not load the data this way! You must edit the text file so that only one time-depth set of data will be loaded at one time (ie. first “Check”, then “Shell_1998”).

In Geolog -> Make sure AUGER is open, click the “Connect” button on the main menu

Connect -> Load

Inputs:

Device/File: Choose the file you just created on /mking

Physical Format: FILE

Contractor Format: FLAT_ASCII

Flat Ascii Format: start with auger_joe_time_depth. You may have to edit this format by clicking on “Define”. If you need specific help with this, see Heather, every instance is a little different.

Name Translation: None

Primary Well ID: Well_Name

Tape Name: blank

CLICK OFF “Unit Convert”

Project: AUGER@medusa

OK

In the next screen:

Change the well identifier to the well that you are working with (click on the down arrow on the right side of the small box to get a pull-down menu of the wells to choose from)

Change the set to something that matches your format file (TIME_JOE).

Click the “Start Load” button towards the top of the window.

Open Geolog -> Well and look at your well of interest.

You should see a set called “JOE_TIME” with your data loaded.