Exporting Seismic from Landmark and importing into Geolog

In Landmark: From the OpenWorks menu -> Applications -> PostStack/PAL

Choose your seismic project (aug_psiv) and “Launch”

Click “Input Data” -> SeisWorks Seismic -> Parameters

Input File: “List”, choose ag16psiv, then OK

Choose your Areal Extent (for one tracte: traces = 3510 to 3510)

Click on the “Entire Trace” button, or pick your time range

OK

OK in Input Data window

Click the “Output Data” button -> click off all others except “SEG-Y” -> Parameters

Choose “Disk”

SEG-Y File Disk –> “List”, choose a disk location and name that makes sense (/mking/d1/Projects/AUGER/Data/Seismic/jcv_trace3510.sgy), make sure the file ends with .sgy

Click “Update from Template”, choose auger_psi, then OK

Click OK in SEG-Y Data Output window

Click OK in Output Data window

Click “Run” in PostStack window

You can view what’s going on by clicking Job-> View from the main PostStack  menu, then File -> Monitor.

Importing into Geolog

Connect -> Load

Choose your .sgy file that you just created, it will auto-fill-in the rest of the specifics (no need to create a format file)

Click OFF Unit Convert

OK

Change your well name and then “Start Load”

You can now view the seismic trace in a layout.